3.4.16 Chromatography

AS Link:

3.1.1 – Atomic structure (mass spec)

3.1.3 – Bonding

3.3.6 – Organic analysis (mass spec)

A level link:

3.3.13 - Amino acids, proteins & DNA

Introduction

Chromatography is a collective term for similar techniques that allow the separation and identification of components in a mixture, the simplest form being paper chromatography.

They depend on the principle that a mixture can be separated if it is dissolved in a solvent (often called the eluent) and the solution (called the mobile phase) moves over a solid (the stationary phase).

Separation is achieved because the components of the mixture distribute themselves differently between the two phases depending on their affinity for each phase.

- Mobile phase - carries the soluble components of the mixture, the more soluble the component the faster it moves and so moves further up the plate.

- Stationary phase - components in the mixture adsorbed to surface (weak attraction), the greater the affinity a component has for the stationary phase the slower it will move with the solvent.

If any component does not move it must be insoluble in the solvent used and a different solvent must be found.

If suitable mobile and stationary phases are chosen, a mixture of similar substances can be separated completely.

Definition:

Chromatography is a technique for separating the components of a mixture on the basis of differences in their affinities for a stationary and a mobile phase.

There are three types of apparatus that can be used:

- thin-layer (TLC)

- column

- gas (GC)

Definition:

A chromatograph is the apparatus used for chromatographic separation.

Definition:

Chromatogram is a pattern of separated substances obtained by chromatography.

Required practical 12: Separation of species by thin-layer chromatography

Uses

| Analytical chromatography | Preparative chromatography |
| --- | --- |
| Uses small amounts of material and aims to separate, identify and measure the relative proportions of the components in the mixture. It often used comparison between known samples to allow identification through calculating Rf values or retention time. | Carried out on a large scale and is a technique that purifies through separation and allows the collection of the separated component. Identification can be achieved by measuring retention times. |
| TLC GLC | Column |

Methods of separation

| Partition |  | Adsorption |  |
| --- | --- | --- | --- |
| Solute molecules equilibrate or partition between the two phases: the solid (stationary phase) and solvent (mobile phase). So separation is dependent on solubility. stationary phase ↔ mobile phase |  | Solute molecules become attached to adsorption sites on the stationary phase, strongly adsorbed molecules travel more slowly in the moving phase than those that are only weakly adsorbed. Solubility in the solvent also plays a part. More polar the substance the shorter the distance it will travel Less polar the substance the further distance it will travel. |  |
| Stationary phase | Mobile phase | Stationary phase | Mobile phase |
| Thin layer of non-volatile liquid film held on the surface of an inert solid or within the fibres of a supporting solid matrix. | Gas | Polar solid surface, the surface area is maximised by using finely-divided particles. Either: silica SiO2 alumina Al2O3 | Liquid solvent |
| GLC |  | TLC Column |  |

Thin-layer chromatography (TLC)

This technique is used to separate and identify components in a mixture. It is similar to paper chromatograph in the way it is carried out but the separation method is different.

| Separation: | Adsorption (weak bonding to stationary phase) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Stationary phase: | Thin-layer polar matrix such as silica (SiO2) or alumina (Al2O3) coated on a glass or aluminium or plastic sheet |  |  |  |  |
| Mobile phase: | Solvent or solvent mixture |  |  |  |  |
| Technique: | A concentrated solution of the mixture is applied as a spot along a baseline near the bottom of the plate. This is then dipped in a shallow layer of solvent inside a sealed container; producing an atmosphere that is saturated with solvent vapour. This stops the solvent from evaporating as it rises up the plate. The beaker is often lined with some filter paper soaked in solvent. Separation occurs with the components travelling different distances depending on how strongly they interact with the solid phase and how soluble they are in the solvent. The more polar the component the greater the adsorption and so they travel shorter distance up the plate with the solvent. Components that are more soluble will travel further up the plate. The solvent rises up the plate by capillary action carrying the solute components. It is stopped when the leading edge of the solvent, the solvent front, gets close to the top of the paper. The resulting chromatogram is taken out and dried, often in a fume cupboard due to toxic or flammable fumes from the solvent. The plate is removed and dried when the solvent front has almost reached the top. The Rf values can be calculated and the separated components can be identified. |  |  |  |  |
| Visualisation: | Colourless components can be seen by different methods to make them visible: |  |  |  |  |
|  | - Plates that contain a fluorescent dye in the stationary phase can be placed under a UV light, then draw around the spots with a pencil. |  |  |  |  |
|  | Treating the plates with a locating agent at the end colours components. |  |  |  |  |
|  | - Iodine vapours stick to the components and stain them brown. |  |  |  |  |
|  | - Ninhydrin is used for amino acids; it is sprayed on the plate and stains them purple. |  |  |  |  |

Advantages:

- Quick

- Efficient

- Reproducible (under same conditions)

- Adsorbents (different stationary phases) on the plate can be varied.

Starter: 8.7 - Thin layer chromatography

Identifying the components

Chromatography produces a chromatogram.

To identify each component their retention factor, Rf must be calculated, this is the ratio of the distance travelled up the paper by a component relative to the solvent.

Rf values are always between 0 and 1

Must be quoted to 2 d.p.

Rf   = 	distance travelled by compound

distance travelled by solvent

.

Under standard conditions Rf values can be used to identify compounds. Rf values are dependent on the solvent, stationary phase and the room temperature not the length of the plate or how far the solvent travels. So they can be looked up in data tables in order to identify a component. Keeping these conditions identical can be difficult so a more reliable method is to compare known with unknown on the same chromatogram.

Example CGP267

A sugar solution containing a mixture of three sugars is separated using TLC. The chromatogram is shown.

- Calculate the Rf value of spot X.

Rf  =  distance travelled by spot		 2.5	= 0.24

distance travelled by solvent		10.4

| - The table shows the Rf values of three sugars under the conditions used in the experiment. Use the table to suggest a sugar present in spot X. Spot X has an Rf value of 0.24 which matches with the Rf value for fructose. | Sugar; Rf value. Glucose; 0.20. Fructose; 0.24. Xylose; 0.30. |
| --- | --- |

Two-dimensional chromatography

In some cases, it’s difficult to get complete resolution (separation) of the components, this can be resolved by repeating the process using a different solvent and rotating the paper through 90o to produce a two-dimensional chromatogram.

|  | Chromatogram rotated 90o and run again using a differnet solvent. This give a better resolution if the componets are close togther. Two Rf values obtained for each component. |  |
| --- | --- | --- |

Column chromatography

This is a simple technique, often used to separate and purify individual organic compounds from a mixture.

| A glass tube is filled with the stationary phase usually silica or alumina in powder form to increase the surface area. An alternative stationary phase is ion exchange resin in the form of small granules. A filter or plug is used to retain the solid in the tube. A solvent is then added to cover the solid. |  |  |  |
| --- | --- | --- | --- |
|  |  | Packed column |  |
| The mixture to be analysed is dissolved in a minimum of a solvent and added to top the column. A solvent or mixture of solvents is then run through the column, this is called elution. The components travel down the column and the time for each component to reach the end of the column is recorded; this is known as the retention time. |  |  | less polar weakly adsorbed shorter retention time |
|  |  | Column after a mixture has been separated |  |
| Separation: | Adsorption (weak bonding to stationary phase) |  |  |
| Stationary phase: | Finely-divided silica or alumina gel |  |  |
| Mobile phase: | Organic solvent pure solvent or a mixture (the eluent) |  |  |
| Technique: | As in TLC, the solubility of a substance in the solvent and how strongly it is adsorbed onto the stationary phase is important as this determines the retention of each component through the column. The component will move faster down the column if it is more soluble in the solvent and not as strongly adsorbed to the solid phase. The most strongly adsorbed components take the longest time to flow through the column. The more polar the molecule the slower it travels down the column and so the greater the retention time. The solution emerging from the column is called the eluent and is collected as a series of fractions which can be collected and analysed separately. They can be also be identified using their retention time. Definition: Retention time is the time each component remains in the column Coloured substances can be seen through the glass wall as moving bands. Process can be speeded up by forcing the solvent through the system under pressure – called flash chromatography. |  |  |
| Visualisation: | Colourless components can be visualised using a UV light |  |  |

Videoclip: https://www.youtube.com/watch?v=P8Mmpb4OShw

Advantages:

- Fairly large amounts can be separated

- Separated samples can be isolated and collected

Application: CGP268 PQ1-2

Fact recall: CGP268 Q1-4

Gas chromatography

This is one of the most important modern and powerful techniques for the separation and identification of mixtures of volatile compounds. Just like other chromatography methods it consists of a mobile phase and stationary phase.

The mobile phase this time is a continuous steady flow of carrier gas passes through a column. The faster it flows the faster the analysis, but lower the resolution (the poorer the information).

| The stationary phase is a silica or alumina powder, coated with a viscous non-volatile liquid such as oil, which is either packed into or coated on the inside of a long capillary tube, known as the column. These can be between 60-100m long and very thin (0.5-1.0mm diameter). |  |  |  |
| --- | --- | --- | --- |
| The column is coiled to save space and placed in an oven; this allows different temperatures to be used. It’s usually carried out under pressure or at a high temperature. |  |  |  |
|  |  | Column inside GC oven |  |
| Schematic diagram of GC |  |  |  |
|  |  |  | Outside of GC |
| Separation: | Partition (solubility in stationary & mobile phase) |  |  |
| Stationary phase: | Long, narrow-bore stainless steel or glass coiled capillary tube packed with an inert powder (silica or alumina) coated with a film of non-volatile liquid. |  |  |
| Mobile phase: | Inert gas – helium, argon or nitrogen (the eluent) |  |  |
| Technique: | The vaporised sample is injected into the port at the head of the column and the components are carried through the system and appear later, in sequence, at the exit. Solubility of the components in the non-volatile liquid (on stationary phase) is a major factor. In the separation. A component that is highly soluble in the liquid phase takes considerably longer to elude (time to travel through the column) from the column than one having a low solubility. It will have a longer retention time. Each component has a characteristic retention time that depends on: - the nature of the stationary phase - the operating temperature - the flow rate of the carrier gas - the length of the column. The retention times can be measured and compared to standards to identify the components. |  |  |

| Detection: | A detector is used to identify each component as it exits the column. The flame ionisation detector is the most popular detector because it is reliable and sensitive. Analytes combust in a hydrogen-air flame as they exit the column, producing carbon ions that induce a current. This is linked to a recorder and retention time of each component is recorded. It appears as a peak on a chart called the chromatogram. If standards also used, then the retention times can be used to identify the components. The area of the peaks are proportional to the relative amount of the component. |
| --- | --- |
| Chromatogram: | This chromatogram shows the retention times and the area gives the amount of each component. |
| Retention times: |  |

Advantages:

- Used to measure very small quantities

- Distinguish between closely related molecules.

Uses:

- Forensic analysis e.g. crime scene samples

- Drug use analysis e.g. urine or blood samples looking for illegal drugs

- Environmental analysis.

Gas chromatography mass spectroscopy (GCMS)

| A very powerful type of GC is gas chromatography-mass spectrometry (GCMS), which is gas chromatograph coupled to a mass spectrometer, where the mass spectrometer is used as the detector for the GC. The separated components are ionised as they emerge from the column and both the identity and quantity are determined by mass spectrometry. |  |
| --- | --- |

Advantages:

- Separated components can be positively identified.

Comparing the techniques

|  | TLC | Column | GC |
| --- | --- | --- | --- |
| Mobile phase | Solvent or solvent mixture | Organic solvent | Inert carrier gas – helium or nitrogen |
| Stationary phase | Thin-layer polar matrix such as silica gel or alumina | Finely-divided silica gel or alumina gel | Capillary tube packed with an inert powder coated with a film of non-volatile liquid |
| Method of separation | Adsorption | Adsorption | Partition |

Video: Chemical analysis techniques – Chromatography

Sheet: Chromatography

Application: CGP270 PQ1-3

Fact recall: CGP270 Q1-4

Exam questions: Oxford p242-243 Q1-3